# Miniature Dozer Control & Payment Integration - Component Specification

This document outlines the recommended hardware components, minimum specifications, and connection guidelines for integrating a timed control and payment system into the miniature dozer.

# 1. Required Components

Microcontroller (Control Unit)

Recommended: ESP32
Minimum Requirements:
- 3.3V logic
- At least 2 GPIO pins (relay + optional display)
- Analog input (for battery sensing)
-  WiFi capability

Relay Module (Power Control)

Recommended: 5V Relay Module
Minimum Requirements:
- Rated for at least 10A (preferably 20–30A for safety margin)
- Screw terminals
- 5V trigger input

Voltage Sensor Module

Recommended: 0–25V Voltage Sensor Module
Minimum Requirements:
- Supports battery voltage range (up to 12–24V)
- Analog output compatible with ESP32 (0–3.3V)

DC-DC Converter

Recommended: Buck Converter (12V → 5V)
Minimum Requirements:
- Input range covering battery voltage
- Stable 5V output
- At least 1A output current

LED Timer Display (Optional)

Recommended: TM1637 4-digit display
Minimum Requirements:
- 4-digit display
- Compatible with microcontroller GPIO

Emergency Stop Button

Minimum Requirements:
- Normally closed (NC) or interrupt-style switch
- Cuts power immediately

Fuse

Minimum Requirements:
- Inline fuse rated slightly above operating current

# 2. Connection Overview

Battery (+) → Fuse → Relay → Receiver (+)
Battery (-) → Receiver (-)

Battery (+/-) → Voltage Sensor
Voltage Sensor → ESP32 (analog pin)

Battery → DC-DC Converter → ESP32 (5V)

ESP32 → Relay (control signal)

# 3. System Behavior

- When system is enabled: relay ON → receiver powered → dozer operates
- When timer ends or fault occurs: relay OFF → receiver loses power → dozer stops
- ESP32 continuously monitors battery voltage and can block operation if battery is low

# 4. Control Chip (ESP32) Configuration

The ESP32 does NOT need to be pre-programmed by the manufacturer.

Recommended setup:
- Manufacturer installs ESP32 physically inside the unit
- Power and GPIO pins are wired and accessible
- No firmware required at installation stage

Final configuration:
- System integrator will upload software later via USB or WiFi

Important:
- Ensure access to ESP32 USB port or programming pins
- Ensure GPIO pins are accessible

# 5. Notes for Manufacturer

- Do not modify motor controller (ESC) or remote system
- Only integrate control at receiver power level
- Ensure all wiring is secure and uses proper connectors
- Keep system modular and removable